Department of Electrical Engineering

| Faculty Member: Miss Rafia | Dated: 11-3-25 |
| --- | --- |
| Semester: 2nd | Section: ds-2a |

Group No.:

EE-241: Digital Logic Design

Lab 07:  Design a display system of a rolling dice

|  |  | PLO4/CLO4 | PLO4/CLO4 | PLO5/CLO5 | PLO8/CLO6 | PLO9/CLO7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Reg. No | Viva / Lab Performance | Analysis of data in Lab Report | Modern Tool Usage | Ethics and Safety | Individual and Team Work | Total marks Obtained |
|  |  | 5 Marks | 5 Marks | 5 Marks | 5 Marks | 5 Marks | 25 Marks |
| AILYA ZAINAB | 523506 |  |  |  |  |  |  |
| IMAN NAEEM | 525378 |  |  |  |  |  |  |
| LAIBA NASIR | 510419 |  |  |  |  |  |  |
| LUQMAN SHEHZAD | 507599 |  |  |  |  |  |  |

Lab7: Design a system that display from 1 to 6. It displays the result on a dice. The dice has seven lights

This Lab Activity has been designed to practice the use of basic gates for designing a system

- Simplification of Combinational Circuits

- Design and Implementation of a design a system to display dice values.

- Values-Segment Decoder for Selected Digit Display

- There are related questions at the end of this activity. Give complete answers.

Use diagrams if needed for clarity.

PreLab Task (2 marks)

Design a system that display from 1 to 6 (ONLY)i.e. it shows no output in case of illegal input like 0 and 7. It displays the result on a dice. The dice has seven LEDs  a,b,c,d,e,f,g,  placed in H shape pattern as shown on the diagram below

a

b

c

f

e

g

d

A 1 for each segment ( a, b, c, d, e, f, g) indicates that it is lit (on); and a 0 that it is off. The arrangement is the six numbers on a dice are shown below; where the darken circles depicts LED is ON. (Make sure to switch off all the lights in Don’t care case)

7 bit output

3 bit Input

6

5

4

3

2

1

- Design a driver that produces the seven LED (a, b, c, d, e, f, g) to drive the display.

- Complete the following table.	(Make sure to switch off all the lights in Don’t care case)	(First 3 inputs are filled for guidance)

|  | Inputs (Binary) |  |  | Outputs (7 LEDs on Dice Display) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| #m | A | B | C | a | b | c | d | e | f | g |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 2 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 3 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| 4 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 5 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 |
| 6 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 |
| 7 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |

- Write minimum possible functions to realize outputs (Either using k-mapping/ or minimization of Boolean function) (Make sure to switch off all the lights in Don’t care case). Show and get verified the minimized Boolean Function expressions to Lab Engineer before implementation.

Hint: You will get 7 output expressions. Output expression for ‘a’ and ‘g’ will be same. Output expression for ‘b’, and ’f’ will be same. Output expression for ‘c’ , and ‘e’  will be same.

Simplified Expressions

| Variable |  | Expression |
| --- | --- | --- |
| a | g | (A ⊕ B) + BC’ |
| b | f | A(BC’) |
| c | e | A(B’+C’) |
| d | - | C(A’+B’) |

- Draw the complete logic circuit diagram of the system from simplified equations (Proteus)

DICE FORM OUTPUT:

Lab Task 1 (5 marks)

- Implement the designed logic circuit on hardware. Utilize your creativity to make the Dice display model

|  | Inputs (Binary) |  |  | Outputs (7 LEDs on Dice Display) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| #m | A | B | C | a | b | c | d | e | f | g |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 2 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 3 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| 4 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 5 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 |
| 6 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 |
| 7 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |

Hardware Implementation

PROTEUS:

OUTPUT:

Lab Task 2 (3 marks)

- Write Verilog code to realize the design using dataflow model. Design also test bench to check the valid outputs. Include all the snap shots and Verilog code in the report.

CODE:

module task1(a, b, c, d, e, f, g, A, B, C);

input A, B, C;

output a, b, c, d, e, f, g;

assign a = (~A & B) | (A & ~B) | (B & ~C);

assign g = (~A & B) | (A & ~B) | (B & ~C);

assign b = A & (B & ~C);

assign f = A & (B & ~C);

assign c = A & (~B | ~C);

assign e = A & (~B | ~C);

assign d = C & (~A | ~B);

endmodule

module testbench;

reg A, B, C;

wire a, b, c, d, e, f, g;

task1 t1(a, b, c, d, e, f, g, A, B, C);

initial

begin

#100 A = 0; B = 0; C = 0;

#100 A = 0; B = 0; C = 1;

#100 A = 0; B = 1; C = 0;

#100 A = 0; B = 1; C = 1;

#100 A = 1; B = 0; C = 0;

#100 A = 1; B = 0; C = 1;

#100 A = 1; B = 1; C = 0;

#100 A = 1; B = 1; C = 1;

end

Endmodule

WAVE FORM:

Observations/Comments:

In this lab we implemented the logic to design a display system of a rolling dice using:

- Basic gates (AND, NOT and OR)

- Using XOR

We implemented the LEDs (placed in an H - shape) at the receier end of the circuit and wrote the Verilog code for the circuit and verified the outputs.